Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#03)

Apply Behavioral-level Verilog to Design and Simulate Combinational Modules

Student Name: Muhammad Ashnab Safdar Roll No: 24I-6500 Section: CE-A

Date performed: 1/9/2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 1:

Description:

Write a Verilog module that accepts two 4-bit numbers and a control signal and adds or subtracts the numbers, depending on the value of the control signal. Make a testbench to simulate and check addition and subtraction.

Code:

module adder_sub(A,B, ctrl, Cout); input [0:3]A; input [0:3]B; input ctrl; output reg [0:4] Cout; always @ (*) begin case(ctrl) 1'b1:Cout=A+B; 1'b0:Cout=A-B; endcase end endmodule module testbench_adder_sub(); reg [0:3] a; reg [0:3] b; reg ctrl; wire[0:4] result; adder_sub uut( a,b,ctrl,result); initial begin a = 0; b = 0; ctrl = 0; #10; a = 4'd5; b = 4'd3; ctrl = 1'b0; #10; a = 4'd12; b = 4'd6; ctrl = 1'b0; #10; a = 4'd9; b = 4'd4; ctrl = 1'b1; #10; a = 4'd4; b = 4'd6; ctrl = 1'b1; #10; end endmodule

Output:

Task 2:

Description:

Write a testbench to simulate and test both select cases for a given Verilog module with two 1-bit data inputs (A, B), a 1-bit Select line and a 1-bit output Y such that the module outputs A & B when Select=0 and A ¦ B when Select=1

Code:

module mux2x1(A,B, ctrl, Cout); input A; input B; input ctrl; output reg Cout; always @ (*) begin case(ctrl) 1'b1:Cout=A¦B; 1'b0:Cout=A&B; endcase end endmodule module testbench_mux2x1(); reg a; reg b; reg ctrl; wire result; mux2x1 uut (.A(a),.B(b),.ctrl(ctrl),.Cout(result)); initial begin ctrl = 1'b0; a = 1'b0; b = 1'b0; #10; ctrl = 1'b0; a = 1'b0; b = 1'b1; #10; ctrl = 1'b0; a = 1'b1; b = 1'b0; #10; ctrl = 1'b0; a = 1'b1; b = 1'b1; #10; end endmodule

Output:

Task 3:

Description:

Write a Verilog module that compares two 2-bit binary numbers and returns three signals: equal, greater-than, less-than that indicate the results of the comparison. Write a testbench to simulate and verify all combinations.

Code:

module comparator_2bit( input [1:0] A, input [1:0] B, output reg eq, // 1 if A == B output reg gt, // 1 if A > B output reg lt // 1 if A < B ); always @(*) begin if (A == B) begin eq = 1; gt = 0; lt = 0; end else if (A > B) begin eq = 0; gt = 1; lt = 0; end else begin eq = 0; gt = 0; lt = 1; end end endmodule module comparator_2bit_testbench; reg [1:0] A, B; wire eq, gt, lt; integer i, j; comparator_2bit uut (.A(A),.B(B),.eq(eq),.gt(gt),.lt(lt)); initial begin for (i = 0; i < 4; i = i + 1) begin for (j = 0; j < 4; j = j + 1) begin A = i[1:0]; B = j[1:0]; #10; end end end

Output: